Quantization Comparison Report

Tanh + AvgPool vs ReLU + MaxPool

# 1. Overview

This report compares five numerical formats—Float32, Fixed16 <13,3>, Ultra-low (1,2), INT8, and INT4—using two model configurations. The comparison considers classification accuracy, quantization error, and memory usage.

Configurations: Tanh + AvgPool and ReLU + MaxPool.

# 2. Quantization Comparison — Tanh

| Format | Source | Accuracy (%) | Max Error | Mean Error | RMSE | Memory (bytes) |
| --- | --- | --- | --- | --- | --- | --- |
| Float32 | Baseline | 98.40 | 0.00000 | 0.00000 | 0.00000 | 246,824 |
| Fixed16 <13,3> | Paper | 97.20 | 0.06250 | 0.03122 | 0.03605 | 123,412 |
| Ultra-low (1,2) | Our test | 95.40 | 0.12499 | 0.06121 | 0.07105 | 30,853 |
| INT8 | Our ext | 98.40 | 0.00241 | 0.00096 | 0.00111 | 61,706 |
| INT4 | Our ext | 97.80 | 0.04379 | 0.01739 | 0.02013 | 30,853 |

# 3. Quantization Comparison — ReLU

| Format | Source | Accuracy (%) | Max Error | Mean Error | RMSE | Memory (bytes) |
| --- | --- | --- | --- | --- | --- | --- |
| Float32 | Baseline | 98.00 | 0.00000 | 0.00000 | 0.00000 | 246,824 |
| Fixed16 <13,3> | Paper | 97.20 | 0.06250 | 0.03144 | 0.03622 | 123,412 |
| Ultra-low (1,2) | Our test | 69.60 | 0.12500 | 0.06057 | 0.07038 | 30,853 |
| INT8 | Our ext | 98.20 | 0.00277 | 0.00098 | 0.00115 | 61,706 |
| INT4 | Our ext | 98.00 | 0.05030 | 0.01784 | 0.02096 | 30,853 |

# 4. Combined Comparison

| Format | Source | Tanh Accuracy (%) | ReLU Accuracy (%) | Difference (%) | Tanh RMSE | ReLU RMSE | Memory (KB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Float32 | Baseline | 98.40 | 98.00 | -0.40 | 0.00000 | 0.00000 | 241.04 |
| Fixed16 <13,3> | Paper | 97.20 | 97.20 | +0.00 | 0.03605 | 0.03622 | 120.52 |
| Ultra-low (1,2) | Our test | 95.40 | 69.60 | -25.80 | 0.07105 | 0.07038 | 30.13 |
| INT8 | Our ext | 98.40 | 98.20 | -0.20 | 0.00111 | 0.00115 | 60.26 |
| INT4 | Our ext | 97.80 | 98.00 | +0.20 | 0.02013 | 0.02096 | 30.13 |

# 5. Accuracy Graph

The following graph compares accuracy across the five quantization formats for the two model configurations: tanh + AvgPool and ReLU + MaxPool.

# 6. Key Findings

Float32: Provides the baseline accuracy of 98.4% for tanh + AvgPool and 98.0% for ReLU + MaxPool, but requires the highest memory at 241.04 KB.

Fixed16 <13,3>: Reduces memory by roughly half compared with Float32, while maintaining 97.2% accuracy for both configurations. Its RMSE is around 0.036.

Ultra-low (1,2): Uses only 30.13 KB, but suffers a major accuracy degradation with ReLU + MaxPool, falling to 69.6%. With tanh + AvgPool it reaches 95.4%.

INT8: Offers the strongest overall accuracy-to-memory trade-off. It reaches 98.4% on tanh + AvgPool and 98.2% on ReLU + MaxPool while using only 60.26 KB.

INT4: Matches the Ultra-low format's 30.13 KB memory footprint while achieving much better accuracy: 97.8% with tanh + AvgPool and 98.0% with ReLU + MaxPool.

# 7. Conclusion

Based on the reported results, INT8 is the most balanced choice when both accuracy and memory efficiency are important. It retains essentially the Float32-level accuracy while reducing memory from 241.04 KB to 60.26 KB. If the primary objective is minimum memory, INT4 is considerably more reliable than the tested Ultra-low (1,2) format, especially for the ReLU + MaxPool configuration. The Ultra-low (1,2) representation appears highly sensitive to the activation/pooling combination and is therefore not the preferred option for the ReLU configuration.

# 8. Metric Notes

Accuracy: Percentage of correctly classified samples.

Max Error: Largest absolute difference between the reference and quantized values.

Mean Error: Average absolute quantization error.

RMSE: Root Mean Square Error; lower values indicate smaller overall numerical deviation.

Memory: Reported storage required for the corresponding model representation.